DESIGN MODIFICATIONS TO THE CRM INCORPORATING XILINX  E-SERIES DEVICES

General

The pre-production version of the CRM board has been used as the test bed for these modifications.  Three integrated circuits, U21, U25, U72 have been changed to Xilinx E-series devices.  Two other integrated circuits, U14 and U15 are already fitted as Xilinx E-series devices and no change has been made to these.

The gate array designs were targeted at the new devices and recompiled.  All the designs required some rework in order to achieve the time delays that were considered necessary.  The extent of the rework varied from design to design.  Four designs were affected because one of the devices has two versions of the design (MMUX and DMUX).

The modifications have been tested for interface to the PCM and DMM’s.  The Station Unit rack was populated with 2 DIM’s, 4 TRM’s, 1 CRM, 1 PCM, 1 DCM, 2 DMM’s, 1 SUIM.  These tests indicate that the interface is satisfactory.  However a full Station Unit FAT will be required to confirm that the modifications are fully functional.

The bit stream files for the new gate arrays are larger and the software has been modified to account for this.  The blast files for programming the flash ROMs are also larger.  This has necessitated changing to 2MBIT flash ROMs.  A link has been added to LK16 to connect the extra address line.

The E-series devices are much faster than the original XC4000 devices with much faster rise and fall times.  This caused particular problems with the Track Clock output of U21 in the DMUX mode.  It was found necessary to slow these edges with a 100pF capacitor to ground.

Hardware

The hardware modifications are as follows:-

1) Change U21 from XC4004A-5PC84C to

XC4005E-3PC84C.

2) Change U25 from XC4005H-5PG223C to

XC4013E-4PG223C.

3) Change U72 from XC4005H-5PG223C to

XC4013E-4PG223C.

4) Change U42 and U43 from INTEL P28F010-65

INTEL P28F020-70.

5) Add a link between LK16, pin1 and LK16, pin8.

6) Add a 100pF capacitor between U21, pin40 and U21, 	 	               pin52.  U21, pin52 is a ground pin.

Gate Array Design

The following gate array designs have been modified:-

1) 6010010 which resides in U21.

2) 6110010 which resides in U25 dependant on the CRM     		    configuration (MMUX).

3) 6310010 which resides in U72.

4) 6410010 which resides in U25 dependant on the CRM     		     configuration (DMUX).

Software

The following files were changed to cater for the longer bit 		 streams:-

1) MMUX_XIL.H - the new include file for the MMUX 			   configuration.

2) DMUX_XIL.H - the new include file for the DMUX 			    configuration.

3) FPGADATA.S - to redefine the size of the FPGA data     		    storage.

4) PROG_XIL.CC - to change the terminal count of the 			    loops which manipulate the FPGA data and the type of 		    the variable which is used (UWORD to ULONG).